# Lab 2: Spec-Driven Design Creation Prototype on the Kria KV260 Vision AI Starter Kit

2026.1 EA

## Abstract

In this lab, you will use a hardware specification and a small custom RTL module to create a complete Vivado™ design for the AMD Kria™ KV260 Vision AI Starter Kit through prompts entered in VS Code chat. The workflow reads an engineer-authored hardware specification, creates the Vivado project, assembles the block design, adds the custom AXI-Stream filter as a module reference, assigns addresses, builds through a bitstream, and reports timing. The resulting design implements a DMA loopback through BRAM, allowing verification with devmem2 without kernel drivers, PMODs, or external I/O. The optional hardware phase then uses manual bitstream loading and simple register-level DMA setup to validate the end-to-end data path on the target board.

## Objectives

After completing this lab, you will be able to:

Use a hardware specification document to drive AI-assisted design creation for a Kria KV260-Vivado project.

Build a complete block design containing Zynq™ UltraScale+™ PS, AXI DMA, BRAM, SmartConnect, Timer, interrupt aggregation, and a custom AXI-Stream filter module reference.

Interpret the generated address map and understand how the CPU and DMA share access to the BRAM buffer regions used for loopback testing.

Complete implementation through bitstream generation and review the expected timing results for the 100 MHz target design.

Optionally validate the design on hardware using manual bitstream loading and devmem2-based DMA loopback verification.

## Introduction

This lab demonstrates a spec-driven design creation workflow in which the engineer writes the hardware intent and the AI assistant implements the Vivado project from that specification. The supplied hardware specification defines the target device, required IP blocks, parameters, address map, and data path, while the custom RTL module axis_filter.v provides the AXI-Stream processing stage inserted between the DMA MM2S and S2MM channels. The design target is a BRAM-backed DMA loopback on the Kria KV260, where the processor writes data into one BRAM region, AXI DMA reads that data into the AXI-Stream path, the custom filter passes it through, and the DMA writes the result back into a second BRAM region.

The workflow is intentionally simple and uses only standard, well-documented IP blocks. This makes the lab suitable for demonstrating how AI converts a good hardware specification into a working design while keeping the scope bounded to a design that can be implemented reliably from the available specification and RTL assets.

## General Flow

| Step | Description |
| --- | --- |
| Step 1 | Environment readiness and workspace preparation |
| Step 2 | Spec-driven project build through bitstream |
| Step 3 | Generated design review and address-map inspection |
| Step 4 | Optional hardware preparation and manual bitstream loading |
| Step 5 | Optional DMA loopback verification on hardware |

Environment Readiness and Workspace Preparation	Step 1

1-1. Open the lab workspace.

Open the design-creation-prototype folder in VS Code.

In the CloudShare environment, browse to /home/amd/training/vivado-ai-assistant-examples, select the design-creation-prototype folder, and click Open.

This workspace includes the engineer-authored hardware specification in spec/hardware_spec.md, the custom RTL module in src/axis_filter.v, and the prompt library used by the AI assistant.

1-2. Verify the Vivado AI assistant environment.

Ensure that the Vivado MCP Server connection is configured correctly and that the AI chat panel is available in VS Code.

To verify the extension, open the Extensions sidebar in VS Code and search for Vivado. The Vivado AI Extension should appear in the results.

The design-creation flow assumes that project creation, block-design assembly, address assignment, synthesis, implementation, and bitstream generation are initiated through natural-language prompts rather than through manual Tcl entry in the Vivado IDE console.

1-3. Review the intended design architecture.

The target design is a DMA loopback through BRAM on the Kria KV260. The CPU writes test data into a BRAM source region, AXI DMA MM2S reads that region, sends the data through the custom axis_filter module in the AXI-Stream path, and AXI DMA S2MM writes the result back to a different BRAM destination region. Verification uses devmem2 only, so no kernel driver development or external I/O path is required.

Spec-driven Project Build Through Bitstream	Step 2

2-1. Build the complete design in batch mode (or use Step 2-2 for GUI mode).

Enter the following prompt in VS Code chat:

Create a Vivado block design for the Kria KV260 according to the hardware specification in spec/hardware_spec.md. Add the custom RTL module from src/axis_filter.v as a module reference. Build through bitstream and report timing.

This prompt instructs the assistant to read the hardware specification, create a Vivado project targeting the xck26-sfvc784-2LV-c device using the KV260 board preset, instantiate and configure the required IP, add axis_filter.v as a module reference in the AXI-Stream data path, assign the address map, validate the block design, generate the HDL wrapper, and then run synthesis, implementation, and bitstream generation.

2-2. Build the complete design in GUI mode (optional).

Enter the following prompt in VS Code chat:

Start Vivado tool in GUI mode, then create a Vivado block design for the Kria KV260 according to the hardware specification in spec/hardware_spec.md. Add the custom RTL module from src/axis_filter.v as a module reference. Build through bitstream and report timing.

This prompt runs the same design-creation flow but keeps the Vivado IDE GUI open for visual inspection during execution.

2-3. Understand what the assistant creates from the specification.

The project that is generated from the specification includes the Zynq UltraScale+ PS with M_AXI_HPM0_FPD enabled and pl_clk0 running at 100 MHz, an AXI DMA configured for 64-bit data in simple mode with DRE enabled, the custom axis_filter module reference in the MM2S-to-S2MM stream path, an AXI BRAM Controller and Block Memory to provide a shared 16 KB loopback buffer, an AXI SmartConnect interconnect, an AXI Timer, and a Concat block for interrupt aggregation into pl_ps_irq0.

Generated Design Review and Address-map Inspection	Step 3

3-1. Review the generated IP composition.

After the build completes, review the generated design logically rather than interactively editing it. The important point is that the AI-generated block design reflects the intent expressed in the specification and that the standard IP configuration, custom module insertion, and address assignment are all completed without manual fixup.

| Component | Configuration / Role |
| --- | --- |
| Zynq UltraScale+ PS | Board preset applied, M_AXI_HPM0_FPD enabled, pl_clk0 at 100 MHz |
| AXI DMA | 64-bit data width, simple mode, DRE enabled |
| axis_filter | Custom RTL module reference inserted in the MM2S → S2MM loopback path |
| AXI BRAM Controller + Block Memory | 16 KB shared source/destination buffer |
| AXI SmartConnect | 3 SI × 3 MI interconnect for PS and DMA masters |
| AXI Timer | Interval timer with interrupt |
| Concat | Aggregates DMA and Timer interrupts to pl_ps_irq0 |

3-2. Review the generated address map.

| Peripheral | Base Address | Size | Accessed By |
| --- | --- | --- | --- |
| AXI DMA | 0xA000_0000 | 64 KB | CPU only |
| AXI Timer | 0xA001_0000 | 64 KB | CPU only |
| AXI BRAM | 0xA002_0000 | 16 KB | CPU + DMA MM2S + DMA S2MM |

3-3. Review the expected timing outcome.

| Metric | Value |
| --- | --- |
| WNS | +5.488 ns |
| WHS | +0.011 ns |
| Errors | 0 |
| Critical Warnings | 0 |

Optional Hardware Preparation and Manual Bitstream 
Loading	Step 4

Note: This hardware phase is optional. If board access is not available, the core learning objectives of specification-driven design creation, block-design assembly, address-map inspection, and bitstream generation are still satisfied by completing Steps 1 through 3.

4-1. Confirm that board usage is optional.

The source sequence includes KV260 hardware validation, but this lab treats that phase as optional so the design-creation methodology remains useful in environments where a board is not available.

4-2. Manual step—Load the bitstream on the KV260.

This step is manual. Loading the .bit.bin file onto the KV260 through the Linux® FPGA manager is outside the AI-driven Vivado IDE build flow and should be treated as a board-deployment action rather than as part of the VS Code prompt sequence.

Note: The .bit.bin file is not generated by default as part of the implementation flow. Before proceeding with board deployment, ensure you explicitly generate the .bin file by providing an appropriate prompt in VS Code. This step is required for KV260 programming using the Linux FPGA manager.

# From your workstation
scp build/*.bit.bin ubuntu@<KV260_IP>:/home/ubuntu/

# On the KV260
sudo cp /home/ubuntu/*.bit.bin /lib/firmware/
echo 0 | sudo tee /sys/class/fpga_manager/fpga0/flags
echo "kv260_dma_bram.bit.bin" | sudo tee /sys/class/fpga_manager/fpga0/firmware

Optional DMA Loopback Verification on Hardware	Step 5

5-1. Request the verification sequence.

Enter the following prompt in VS Code chat:

The bitstream is loaded on the KV260. What devmem2 commands should I run to test the DMA loopback through BRAM?

This prompt generates a verification sequence that you can use to validate the design. Alternatively, you can run the commands in Steps 5-2 through 5-4 directly to verify the design functionality.

5-2. Write test data into the BRAM source region.

sudo devmem2 0xA0020000 w 0xDEADBEEF
sudo devmem2 0xA0020008 w 0x12345678
sudo devmem2 0xA0020010 w 0xCAFEBABE
sudo devmem2 0xA0020018 w 0xA5A5A5A5

Note: If devmem2 is not installed on the machine, install it using the following command:

sudo apt install devmem2

5-3. Configure and start the DMA transfer.

sudo devmem2 0xA0000000 w 0x00000004
sudo devmem2 0xA0000030 w 0x00000004
sudo devmem2 0xA0000000 w 0x00000001
sudo devmem2 0xA0000030 w 0x00000001
sudo devmem2 0xA0000048 w 0xA0021000
sudo devmem2 0xA0000018 w 0xA0020000
sudo devmem2 0xA0000058 w 32
sudo devmem2 0xA0000028 w 32

5-4. Read back the BRAM destination region.

sudo devmem2 0xA0021000 w
sudo devmem2 0xA0021008 w
sudo devmem2 0xA0021010 w
sudo devmem2 0xA0021018 w

5-5. Observe the BRAM alignment rule.

The BRAM controller uses a 64-bit data width, so only 8-byte aligned addresses such as 0x0, 0x8, 0x10, and so on should be accessed. Accesses to 4-byte-only aligned addresses such as 0x4 or 0xC cause Bus Errors.

## Summary

In this lab, you used an engineer-authored hardware specification and a custom RTL module to generate a complete Kria KV260-Vivado design through prompts entered in VS Code chat. The assistant created the project, assembled the block design, inserted the custom axis_filter module reference, assigned the address map, and built through a bitstream. The optional hardware phase then treated bitstream loading as a manual board-deployment step and used devmem2-based DMA loopback verification to validate the full data path through BRAM and AXI DMA.