# Lab 5: AXIS-ILA and AXIS-VIO Insertion Flow on the Versal VCK190 Evaluation Kit

2026.1 EA

## Abstract

In this lab, you will build a PL-only AXI4-Stream debug pipeline on the AMD Versal™ VCK190 Evaluation Kit and instrument it using the AXIS-VIO and AXIS-ILA cores through prompts entered in VS Code chat. The workflow creates a base block design from a hardware specification and RTL sources, inserts AXIS-VIO for runtime control and monitoring, inserts AXIS-ILA for interface-level waveform capture, and then completes implementation through PDI generation. The design intentionally avoids PS data paths, DMA, and DDR so that the lab remains focused on programmable-logic debug instrumentation and live observation over JTAG.

## Objectives

After completing this lab, you will be able to:

Build a Versal VCK190 project from a hardware specification and RTL sources using a prompt-driven workflow in VS Code.

Insert an AXIS-VIO core to drive stream_enable, pattern_sel, and bypass_enable while monitoring packet_count without manual IP configuration.

Insert an AXIS-ILA core on an AXI4-Stream interface using the correct interface-monitor settings required for Versal device block-design debug flows.

Generate the complete PDI, review implementation timing, and understand the minimal timing impact of the inserted debug cores for this configuration.

Optionally validate the design on hardware and distinguish AI-driven implementation steps from manual board-programming steps.

## Introduction

This lab demonstrates a streamlined hardware-debug workflow for Versal devices using the Vivado™ AI Assistant and bundled block-design debug-core insertion skills. The functional design is a free-running AXI4-Stream pipeline in programmable logic only, built from axis_stream_source, axis_filter, and an AXI4-Stream Data FIFO. Instead of adding a software-controlled data path, all interaction is performed through a JTAG-connected debug infrastructure, which keeps the lab compact and focused on runtime debug instrumentation.

The bundled bd-vio-insertion and bd-ila-insertion skills encode the required Vivado parameter sequencing, interface connections, and Versal device-specific requirements, including the use of AXIS-VIO and AXIS-ILA and the routing of the debug path through the AXI Debug Hub and NoC. The instructional flow is intentionally staged: build the base design first, then insert the VIO, then insert the ILA, and finally implement and optionally validate on hardware.

## Prerequisites

VS Code with the Vivado MCP Server extension connected.

Vivado IDE 2025.2 or later installed and licensed for Versal devices.

Optional hardware access for the validation phase, such as the Versal VCK190 Evaluation Kit connected through JTAG.

## General Flow

| Step | Description |
| --- | --- |
| Step 1 | Environment Readiness and Workspace Preparation |
| Step 2 | Base Versal Device Block-Design Creation Through Synthesis |
| Step 3 | AXIS-VIO Insertion for Runtime Control and Monitoring |
| Step 4 | AXIS-ILA Insertion for AXI4-Stream Waveform Capture |
| Step 5 | Implementation and PDI Generation |
| Step 6 | Optional Hardware Validation |

Environment Readiness and Workspace Preparation	Step 1

1-1. Open the lab workspace.

Open the ila-insertion-flow folder in VS Code.

In the CloudShare environment, browse to /home/amd/training/vivado-ai-assistant-examples, select the ila-insertion-flow folder, and click Open.

This workspace contains the hardware specification, RTL sources, prompt examples, and the two bundled skills used to insert debug cores into the block design.

1-2. Verify the Vivado AI assistant environment.

Ensure that the Vivado MCP Server connection is configured correctly and that the AI chat panel is available in VS Code.

To verify the extension, open the Extensions sidebar in VS Code and search for Vivado. The Vivado AI Extension should appear in the results.

The workflow assumes that the assistant will drive the Vivado flow through natural-language prompts rather than through manual IP insertion in the Vivado GUI.

1-3. Review the intended hardware-debug architecture.

The design you will build is a PL-only AXI4-Stream path of axis_stream_source → axis_filter → AXI4-Stream Data FIFO. AXIS-VIO will drive stream_enable, pattern_sel, and bypass_enable and will monitor packet_count, while AXIS-ILA will observe the AXI4-Stream interface between the filter and the FIFO.

Base Versal Device Block-design Creation Through Synthesis	Step 2

2-1. Build the base design.

Enter the following prompt in VS Code chat:

Build the VCK190 project in GUI mode, from spec/hardware_spec_vck190.md with RTL sources from src/. Run through synthesis only.

This prompt instructs the agent to read the hardware specification, create a Vivado project targeting the VCK190 device xcvc1902-vsva2197-2MP-e-S, add the RTL files as module references, and build the initial block design.

2-2. Review the synthesized base design structure.

| Block | Role |
| --- | --- |
| versal_cips_0 | Provides control, interfaces, and the pl0_ref_clk and reset infrastructure |
| axi_noc_0 | Provides the NoC path used for JTAG-to-PL debug connectivity |
| axi_dbg_hub_0 | Supplies AXI Debug Hub functionality for debug-core discovery |
| proc_sys_reset_0 | Provides synchronized reset handling |
| axis_stream_source_0 | Free-running AXI4-Stream source with stream_enable, pattern_sel, and packet_count |
| axis_filter_0 | Pass-through filter with bypass_enable |
| axis_data_fifo_0 | AXI4-Stream Data FIFO on the output side of the pipeline |

AXIS-VIO Insertion for Runtime Control and Monitoring	Step 3

3-1. Insert the AXIS-VIO core.

Enter the following prompt in VS Code chat:

Insert an AXIS-VIO to control and monitor the streaming pipeline. Output probes: stream_enable (1-bit, init 0), pattern_sel (2-bit, init 0), bypass_enable (1-bit, init 1). Input probe: packet_count (32-bit). Validate the design.

This prompt activates the bd-vio-insertion skill. The assistant creates an AXIS-VIO core with three output probes and one input probe, connects the probes to the control and monitor signals, connects the VIO clock to pl0_ref_clk, and validates the block design.

3-2. Understand the role of the inserted VIO.

| Probe | Connected Signal | Function |
| --- | --- | --- |
| probe_out0 | axis_stream_source_0/stream_enable | Starts or stops the source |
| probe_out1 | axis_stream_source_0/pattern_sel[1:0] | Selects the generated data pattern |
| probe_out2 | axis_filter_0/bypass_enable | Controls filter bypass / gating behavior |
| probe_in0 | axis_stream_source_0/packet_count[31:0] | Monitors packet throughput |

AXIS-ILA Insertion for AXI4-Stream Waveform Capture	Step 4

4-1. Insert the AXIS-ILA core.

Enter the following prompt in VS Code chat:

Insert an AXIS-ILA on the AXI-Stream interface between axis_filter and the FIFO. Use 1024 sample depth. Validate the design.

This prompt activates the bd-ila-insertion skill. The assistant creates an AXIS-ILA core and configures it for interface monitoring by setting C_MON_TYPE = Interface_Monitor, configures the AXI-Stream slot type, connects the monitored interface, and validates the block design.

4-2. Distinguish the ILA role from the VIO role.

| Parameter | AXIS-VIO | AXIS-ILA |
| --- | --- | --- |
| IP | axis_vio:1.0 | axis_ila:1.0 |
| Monitor Type | — | C_MON_TYPE = Interface_Monitor |
| Slot 0 Interface | — | AXI4-Stream (axis_rtl:1.0) |
| Monitored Net | — | axis_filter_0/m_axis → axis_data_fifo_0/S_AXIS |
| Output Probes | 3 (stream_enable, pattern_sel, bypass_enable) | — |
| Input Probes | 1 (packet_count, 32-bit) | — |
| Sample Depth | — | 1024 |
| Clock | pl0_ref_clk (100 MHz) | pl0_ref_clk (100 MHz) |

Implementation and PDI Generation	Step 5

5-1. Generate the PDI.

Enter the following prompt in VS Code chat:

Generate output products, implement, and generate PDI. Report timing.

This prompt instructs the assistant to generate output products for all IP, perform implementation through opt_design, place_design, and route_design, write the final PDI and the debug probes .ltx file, and then report timing.

### Expected Implementation Results

| Metric | Value |
| --- | --- |
| Target Device | xcvc1902-vsva2197-2MP-e-S |
| Clock | 100 MHz (pl0_ref_clk) |
| WNS (Setup) | +6.132 ns |
| WHS (Hold) | +0.017 ns |
| Timing Status | All constraints met |
| PDI Size | ~1.5 MB |

Optional Hardware Validation	Step 6

Note: This step is optional and requires access to supported hardware, such as a VCK190 connected through JTAG. If hardware is not available, the core learning objectives of block-design debug-core insertion, validation, implementation, and timing review are still satisfied by completing Steps 1 through 5.

6-1. Program the board and summarize discovered debug cores.

Enter the following prompt in VS Code chat:

Program the VCK190 with the generated PDI, associate the debug probes file, and summarize the discovered AXIS-VIO and AXIS-ILA cores

6-2. Enable the stream and monitor throughput.

Enter the following prompt in VS Code chat:

Set stream_enable to 1 using the AXIS-VIO and report whether packet_count begins incrementing

6-3. Block and resume the data path.

Enter the following prompt in VS Code chat:

Set bypass_enable to 0, confirm packet_count stops, then restore bypass_enable to 1 and confirm that packet_count resumes

6-4. Capture AXI4-Stream activity with the ILA.

Enter the following prompt in VS Code chat:

Trigger the AXIS-ILA on TVALID = 1 and capture the AXI-Stream interface at the filter-to-FIFO boundary

### Hardware Test Results

| Test | Expected | Result |
| --- | --- | --- |
| stream_enable = 0 → packet_count is 0 | Count stays at 0 | PASS |
| stream_enable = 1 → packets flowing | Count increments (~0x6A000/sec) | PASS |
| bypass_enable = 0 → data blocked | Count stops | PASS |
| bypass_enable = 1 → data resumes | Count resumes | PASS |
| ILA trigger on TVALID = 1 | Captured incrementing TDATA | PASS |

### Sample ILA Capture Data

| Sample | TDATA (hex) | TVALID | TREADY | TKEEP |
| --- | --- | --- | --- | --- |
| 0 | 000000012659d3c5 | 1 | 1 | ff |
| 1 | 000000012659d3c6 | 1 | 1 | ff |
| 2 | 000000012659d3c7 | 1 | 1 | ff |
| … | (incrementing +1) | 1 | 1 | ff |

Optional Shortcut — Single-Prompt End-to-End Build

Enter the following prompt in VS Code chat:

Build the VCK190 project in GUI mode, from spec/hardware_spec_vck190.md with RTL from src/. Insert AXIS-VIO (stream_enable, pattern_sel, bypass_enable outputs; packet_count input). Insert AXIS-ILA on the filter→FIFO AXI-Stream interface, 1024 depth. Build through PDI and report timing.

## Summary

In this lab, you built a Versal VCK190 Evaluation Kit hardware-debug design entirely through prompts entered in VS Code chat. You first created the base PL-only AXI4-Stream pipeline and its supporting Versal device infrastructure, then inserted an AXIS-VIO core to drive control signals and monitor packet_count, then inserted an AXIS-ILA core to capture AXI4-Stream traffic at the filter output and finally generated the complete PDI for implementation review and optional hardware validation.

Appendix — Agent-Executed Assets and Commands (Reference Only)

The learner-facing procedure in this lab is prompt-driven, but the workflow relies on the hardware specification file spec/hardware_spec_vck190.md, the RTL sources in src/, the bundled skills bd-vio-insertion and bd-ila-insertion, and the implementation flow that writes the PDI and .ltx files.